行政院及所屬各機關出國報告

(出國類別 : 實習)

VOIP交換機維運系統研發設備技術

實習報告

服務機關 : 中華電信研究所

出國人    職    稱 : 研究員

姓    名 : 劉戌蒼

出國地區 : 美國 紐約

出國期間 : 92年 11月29日至92年12月12日

報告日期 : 93年1月27日

摘  要

由於利用網際網路傳送語音通信(VOIP)相關技術之突飛猛進，在話音品質及應用領域將日趨成熟，未來勢必成為通信網路之骨幹。本公司已積極規劃引進及建設IP網路應用，本所為研發VOIP相關技術之需求，因此有必要至國外電信機構現場實習，有關VOIP技術之設計應用與維運管理經驗，以提供本公司VOIP規劃建設，維運及研發相關產品之參考。

本次實習係依照本所92年度”VOIP交換機維運系統研發設備”R920288購案合約辦理，於92年11月29日啟程，赴美國Intel公司實習VOIP交換機相關技術，並於92年12月12日完成課程順利返國。

此次實習主要是針對Intel公司的IP Media Gateway(IPT 1200C) 及IP PSTN Gateway(DM/IP481)等系統設備，參與該公司訓練課程、現場實習操作、技術討論、應用實例參訪等，行程緊湊，獲益良多。

經過此次實習，職除了對於Intel公司VOIP相關產品有深入了解外，對於VOIP設計原理及應用趨勢亦有更深確的認識，將可提供本公司VOIP相關技術規範制定、建設規劃及本所研發VOIP利基產品之參考 。

本報告首先簡述此次實習之目的，接著列出主要行程及實習內容，包括Intel公司之VOIP Media Gateway 相關產品之主要特點及系統架構，硬軟體設計，第三部份描述實習心得，最後提出建議供有關單位參考。

VOIP交換機維運系統研發設備技術

出國實習報告

目   錄

- 目的 ……………………………………………………………………3

- 過程 ……………………………………………………………………4

- 心得 ……………………………………………………………………11

- 建議 ……………………………………………………………………12.

VOIP交換機維運系統研發設備技術

出國實習報告

- 目的

由於利用網際網路傳送語音通信(VOIP)相關技術之突飛猛進，在話音品質及應用領域將日趨成熟，未來勢必成為通信網路之骨幹。本公司已積極規劃引進及建設IP網路應用，本所為研發VOIP相關技術之需求，因此有必要至國外電信機構現場實習，有關VOIP技術之設計應用與維運管理經驗，以提供本公司VOIP規劃建設，維運及研發相關產品之參考。

- 過程

  - 行程

日期                      地點                  工作

92年11月 29~30日        桃園- 紐約            去程

92年12月1~10日         紐約                   Intel公司實習

VOIP設備參訪

92年12月11~12日         紐約- 桃園            回程

  - 內容

本次實習主要係針對Intel公司IP Media Gateway(IPT 1200C) 及IP PSTN Gateway(DM/IP481)等系統設備，參與該公司訓練課程、現場實習操作、技術討論、應用實例參訪等，茲摘要如下 :

2.2.1  IP Media Gateway(IPT 1200C)中介閘道器

2.2.1.1. 主要特點

IP Media Gateway(IPT 1200C)主要特點如下:

- 具備Open, standards-based platforms

- 具備Scalable,carrier-grade solutions

- 支援672 Port 之IP電話路由,以發展具擴充性系統

- 提供乙太網路介面

  - Dual 100Base-TX

  - Dual 100Base-T

  - Provide indicators on the front panel

  - Provide Ethernet interface Link Status

- 傳送編碼器( Transcoding)

  - G.711 A-Law and mu-Law

  - G.723.1 (5.3 Kbps and 6.3 Kbps)

  - G726(Supported via IP Media Gateway)

  - G729 Annex A

- 靜音抑制(Silence Suppression VAD)

-  Ability to detect the absence of audio

- 雙音複頻信號處理 (DTMF Processing)

  - Ability to generate DTMF to and receive notification

  - From IP side via UII alphanumeric message (out of band)

  - From TDM side inband

  - Inband DTMF pass-through

  - RFC 2833 pass-through

- 服務品質(Quality of Service QOS)

  - Support the setting and retrieving of QoS

  - Setting and retrieving of QoS threshold and handling of QoS alarm

  - QoS threshold support : Lost packets, jitter, Roundtrip latency

  - Support the type of services( ToS)

- 支援IP傳送語音 (Voice over IP support)

  - RTP 處理

  - RTCP處理

  - Jitter 緩衝器管理

  - Packet loss concealment

- 傳真功能

  - 經由UDP送收傳真信號

  - Notification of audio to Fax and Fax to audio

  - Support T.38 Fax relay

- 具備H.323 規約Stack

  - 支援host-based H.323 Protocol stack

  - Establishing calls over IP network

  - Using the Global Call API for call control

- 具備SIP規約Stack

  - Support the host-based SIP RADVISION

  - Establishing calls over IP network

  - Using the Global Call API for call control

- 具備SNMP管理功能

  - Monitoring and control is provided

  - Provide Administration Software

- 具備應用界面( Application interface)

  - Support the R4 Programming environment

  - Global Call API for call control

  - IP Media Library(IPML) API for media management

- 回音消除 (Echo cancellation)

  - Provided on incoming TDM data on a per channel basis

  - The tail length can be set for each channel

  - The following tail length are supported

  - 8 ms

  - 16 ms

  - 32 m

2.2.2 IP PSTN Gateway(DM/IP 481)電話網路閘道器

2.2.2.1. 主要特點

IP PSTN Gateway(DM/IP 481)主要特點如下:

- 具備 Equiped wish  a PSTN network front end

- 具備connected via  NetSructure platform to the CT Bus

- 具備Single board digital IP to PSTN Gateway application

- 具備Conferenceing Resource

  - Shareable conference resource

  - Run time add ane remove conference

  - Active talker notification

  - Automatic notification on adding and removing parties

  - DTMF clamping

  - Volume control

  - Auromatic gain control

  - Echo Cancellation

  - Monitoring function

- 支援RFC2833以發展具擴充性系統

- 支援由IP載送傳真信號

  - Support ITU T.38 for real time fax transmission

  - 經由T.38 閘道器 透過IP-based Network

- 具備H.323規約Stack

  - Support the host-based H.323 RADVISION

  - Includeed system release software

  - Providing IP signaling for establishing calls over IP network

  - Using Global Call API for call control

- 雙音複頻信號處理 (DTMF Processing)

  - Ability to generate DTMF to and receive notification

  - From IP side via UII alphanumeric message (out of band)

  - From TDM side inband

  - Inband DTMF pass-through

  - RFC 2833 pass-through

- 服務品質(Quality of Service QOS)

  - Support the setting and retrieving of QoS

  - Setting and retrieving of QoS threshold and handling of QoS alarm

  - QoS threshold support : Lost packets, jitter, Roundtrip latency

  - Support the type of services( ToS)

- 支援IP傳送語音 (Voice over IP support)

  - RTP 處理

  - RTCP處理

  - Jitter 緩衝器管理

  - Packet loss concealm

- 具備SIP規約Stack

  - Support the host-based SIP RADVISION

  - Establishing calls over IP network

  - Using the Global Call API for call control

- 具備SNMP管理功能

  - Monitoring and control is provided

  - Provide Administration Software

- 具備應用界面( Application interface)

  - Support the R4 Programming environment

  - Global Call API for call control

  - IP Media Library(IPML) API for media management

- 具備Call control implemented on the host, RTC on board

- 提供Split call control via the IP Media Library API

- 支援Global call API support for IP and PSTN

- 提供FCD file generation utility

- 支援Flexing routing  (exportable voice resource)

- 具備Standard Internet protocol including TCP/IP,UDP and RTP/RTCP

- 具備與解碼器全雙工通信功能

- 具備Coder support including GSM,G.729

- 具備Microsoft Netmeeting and Vocal tec

- 提供IP switching bridge two calls with minimum latency

- 提供Type of service (ToS) byte

- 具備Packet redundancy : comply with RFC 2189

- 提供IP media service

- 提供High reliability configuration

- 具備IP Voice stream Resource

- 具備Object ID support when using Non-standard command

- 提供Voice quality parameter and volume control

- 具備呼叫程控功能

  - Continuous Speech Processing (CSP) support

  - Support NFAS on N12 protocals

  - Support R2MF hot down procotal

  - Support PSTN (4 ESS,5 ESS, DMS,CAS) signaling

- 具備語音特性控制功能

  - Gain control

  - DTMF volume control

  - On board PSTN interfaces

  - Support digital interface

  - Support Media load 1 voice and load 2 enhance voice

  - Basic DTMF and MF detect

2.2.3. 系統發展軟體( Development Software)

2.3.1. 應用程式庫( API Libraries)

2.3.1.1.新事件服務程式( New event service )程式庫

- Provide an interface for registering any application with the Intel Dialogic event notification framk

- Subsystem for sending asynchronous, system administration events to application

- Generated when activities take place in the system

- Single board stop or start , board removal or insertion

- CT bus line failures and T1/E1 network alarm

2.3.1.2. 語音會議電話(Audio conference)程式庫

- Support development of host-based conferenceing application

- Develop customized audio conferencing services

- Including library functions, divice drivers, and firmware

- Provide conference bridging and monitoring

- Support per party basis of DTMF clamping

- Adjust the listening volume of conference

- Echo cancellation for each active talker

- High port density

2.3.1.3. 連續性語音處理( CSP)程式庫

- Host based automatic speech recognition (ASR)

- High performance each cancellation

- Voice energy detection, barge-in, voice event signaling, pre-speech buffering

- Full duplex operation

- Including library functions, device driver, fireware and demonstration

- Streaming echo-cancelled data to the TDM

- Each canceller tap length of up to 512 taps

- Ability to re-arm the VAD

- More powerful Voice Activity Detector

2.3.1.4. FAX API 程式庫

- 支援Wide variety of Fax applications

- 提供Intel Dialogic 第六版軟體 for CompactPCI an windows

- 支援ITU-T G3 ,V.17, 資料速度 14.4K

- 符合Data transmission encoding scheme with advanced compression

- Polling and turnaround

- 符合Simple header overlay

- 具備Image bit order (MSB/LSB) conversion

- Encoding of color fax imagines

2.3.1.5.整體呼叫程式庫

- Uniform call control interface for developing application

- Protocol operation on Intel NetStructure, DM3 and Springware architecures

- Designed to support a variety of protocols

- Provide a consistent application interface

- Use the same input and output parameters at the application level

- 支援H.323 and SIP host-based stacks

- 符合 Multi-protocol support on Global call devices

- 符合Register with a Gatekeeper or Registrator

- 支援Support DTMF Mode

- 支援 Fax transmission and reception

2.3.1.6 IP中介媒體( Media) 程式庫

- Used to control media on IP devices

- IP Media library (IPML) Provides DialogicR 6.0 & CompactPCI on Windows

- Preferred DTMF Mode, UII Alphanumeric

- Send and receive Fax for T.38

- Using IPML API to build a PSTN-IP gateway

- Media resource management and media resource operation functionality

- Quality of Service(QoS) threshold alarm configuration and status reporting

- Support of Standard Runtime Library(SRL)

2.3.1.7 模組中繼介面應用程式(Modular Station Interface API)

- Provide high density analog station connectivity

- Support up to 120 stations with tone diction and generation

- 支援FSK caller ID transmission

- Conference Management functions to control conferencing features

- Configuration functions to set and retrieve device parameters

- Device Management functions to set and retrieve device parameters

- Diagnostic functions to test devices

- Routing functions to allow communication between time slots on the CT bus

2.3.1.8. 自然建構管理(Native Configuration Manager)應用程式庫

- 提供Interface for developing customized system configuration

- 具備Get the AUID of a board

- 具備Get the board name from and AUID

- Modifying board-level and system-level configuration

- Starting, stopping and checking the status

- Setting the TDM bus clock master fallback list

- Getting stystem software version information

- Cached prompt management

- Enhancement to Multi Frequency signaling

- Enhancement to DV_TPT termination conditions

2.3.1.9 標準執行( Standard Runtime)應用程式庫

- Provide a common interface for event handling and functionality

- Provide the framework for implementing the programming model

- Events are handled in a standard manner

- Support for alternative variant of the extended asynchronous programming

- Support for the synchronous model

- Support for synchronous with SRL callback model

- Support for polled model

- Asynchronous with non-signal callback model

- Extended asynchronous model

- Device event management

- Device information retrieval using ATDV_prefixed functions

2.3.1.10 語音應用程式( Voice API)

- Building a wide range of high-density call processing application

- Voice messaging, interactive voice response,telemarting/call center

- Including tone signaling,global tone detection and generation

- Call progress analysis and voice encoding algorithm selectable

- Cached prompt management

- Interactive multimedia association with ADPCM algorithm

- Using linear coding ,VOX and WAVE file format

- Speed control and volume control

- Transmit/receive analog display service interface

- Perfect call call progress analysis

- Transaction record

- Bulk data buffer sizing

- 心得

  - 近年來由於IP 硬體設備及相關規約標準的日趨成熟,國外通信大廠主力產品均以Internet 應用環境為主.其功能更加完整且價格不斷下降,因此市場競爭非常激烈,在我們研習之Intel 公司通信部門均感受人事異動/精簡之壓力,每位工作人員需全力以赴,才能存活下來

  - VOIP之遠景看好,語音之重要是永恆的,VOIP初期利基在廉價之長途及國際電話,但長期利基應在多媒體化,加值化等新服務更具發展空間

  - 語音通信著重連續性訊流,須具即時特性,因此VOIP 新規範RTP適時產生

  - RTP(Real Time Transport Protocol) 即時傳輸協定,具有16比次之標頭序碼,32 位元之時間截記(Time stamp)以配合靜音抑制, 64位元之Source ID,使二個Port間可以同時進行傳送數個資訊流, RTP附屬於應用層, 路由器並不予處理

  - RTCP(Real time Transport Control Protocol)係監視RTP之運作,並提示報告,即時性(如語音)或交談式的訊務,優先安排於序列之前頭,公平第按序處理,於產生擁塞之前,及早偵出,丟出一些IP標頭所註Class級較低之封包.

  - DiffSer差別式服務,封包進入Edge路由器註以Class參數,骨幹網路路由器依封包Class施以不同等級處理,訊務量在進入點之節制以防擁塞

  - IP Media Gateway(IPT 1200C)主要特點為提供開放及標準的服務平台,並具有高可靠性,可擴充性之IP閘道器.IPT 2000符合下一代網路(NGN)架構需求及具有重大新功能.,在單一CompactPCI電路板可以提供672Channel之電話服務

  - IP PSTN Gateway(DM/IP 481)主要特點為允許語音經由IP傳送(VOIP),透過Intel NetStructure平台至CT Bus. 利用DM/IP Board 接至PSTN網路,完成IP與PSTN介面功能,並可具有會議電話,語音辨識,文字轉語音等多樣化之應用服務.

- 建議

- VOIP隨著服務需求及技術進步,由早期特定使用者以節省國際電話為目的,演進為現今以IP網路傳送語音信號,並結合多媒體通信服務為需求,以致應用面更為廣泛.然而,隨著IP網路建設日趨普及,與傳統PSTN電路交接更加複雜,對於本公司現有網路建設與維運需求,建議儘早規劃本公司整體IP網路發展策略,維持最佳之市場競爭力及滿足客戶需求

- 國外先進電信廠商對於IP技術與產品,不斷推陳出新,本所係研發單位,對於IP新技術之學習與引進,仍須持續進行與積極推展,以技術領先同業,讓公司達到永續經營與成長之目標.