出國報告（出國類別：實習）

研習高效率煤炭氣化技術

服務機關：台灣電力公司

姓名職稱：陳志聖 化學研究專員

派赴國家：美國

出國期間：自97 年 5 月18日至97 年5月  31  日

報告日期：97年7月23日

行政院及所屬各機關出國報告提要

出國報告名稱：研習高效率煤炭氣化技術

頁數  22  含附件：□是■否

出國計畫主辦機關/聯絡人/電話 : 台電人事處/陳德隆/02-23667685

出國人員姓名/服務機關/單位/職稱/電話

陳志聖/台電公司/綜合研究所/化學師/02-80782239

出國類別：□1考察□2進修□3研究■4實習□5其他

出國期間：自97 年 5 月18日至97 年5月 31 日  出國地區：美國

報告日期：97年7月23日

分類號/目

關鍵詞：煤炭氣化、二氧化碳、化學環路程序

內容摘要：（二百至三百字）

本次實習主要在美國俄亥俄州州立大學化學與生物分子工程系研習高效率煤炭氣化技術。

主要研習內容及心得如下:

1. 煤炭是相對經濟、穩定之能源，煤炭發電仍然是未來重要之選項，但環保及二氧化碳問題，使得淨煤技術成為必要之課題。在諸多淨煤技術中，煤炭氣化由於其具多元化應用 及高能源效率之特性，最具發展潛力。先進之氣化技術主要希望以煤炭來替代易受短期因素價格上漲之天燃氣及石油。煤炭氣化重要的優勢如下:

a.能源優勢:可用較差等級的燃煤或工業殘渣如石油焦、瀝青及塔底油等，產生合成氣，代替天然氣，送入複循環發電機組中作為燃氣輪機的燃料，以達到高效率、低發電成本與燃料彈性化的需求。合成氣亦可再製成其他液體燃料及化學品。

b.環保優勢:煤炭氣化係在缺氧狀態，高溫高壓下進行裂解反應，因此產生之氣体量遠小於傳統粉煤燃燒機組之煙氣量，在氣体處理時，無須使用大型之EP及FGD系統(使用較小型之除硫設備即可)，此為煤炭氣化複循環發電技術之一大優點。

c.減碳優勢:淨煤技術中，煤炭氣化化學環路程序（Chemical Looping Process ，CLP），可提升效率及排氣中CO2濃度，以降低CO2捕捉與封存成本，因此受到全世界之重視。

2. 化學環路程序為目前最先進之技術，係利用固体氧化物(例如氧化鐵)取代昂貴之純氧進行氣化，並可將氣化後之合成氣製備成純H2及純CO2，固体氧化物則可循環再利用，為目前效率最高之煤炭氣化技術之一。美國俄亥俄州州立大學化學與生物分子工程系之化學環路程序技術應用於煤炭直接氣化、合成氣之轉化、液態燃料之製備及二氧化碳之分離等。

本文電子檔已傳至出國報告資訊網（http：//report.gsn.gov.tw）

目錄

- 目的  ………………………………………………………………5

- 出國行程  …………………………………………………………6

- 工作記要  …………………………………………………………7

- 感想與建議 ………………………………………………………13

伍、 附   錄    ………………………………………………………15

壹、目的

本次出國之緣由及目的如下:

- 經濟部能源局之能源政策白皮書，即係以「永續」、「安全」、「效率」及「潔淨」為核心目標，在電業、再生能源及節約能源方面，依循「調合能源、環境、經濟3者之均衡發展」、等政策方針加以落實。本公司配合國家政策，在電廠營運上未來可引入先進之高效率煤炭氣化技術以達能源、環境、經濟3者之均衡發展。

- 相對於石油及天然氣，煤炭是相對經濟、穩定之能源，煤炭發電仍然是台電公司及台灣未來重要之選項，但煤炭發電易產生環保及二氧化碳問題，使得淨煤技術成為必要之課題。在諸多淨煤技術中，煤炭氣化技術，由於其具多元化應用及高能源效率之特性，最具發展潛力。煤炭氣化技術中，煤炭氣化化學環路程序，係利用固体氧化物(例如氧化鐵、氧化鈣)取代昂貴之純氧進行氣化，並可將氣化後之合成氣製備成純H2及純CO2，固体氧化物則可循環再利用，可提升能源效率及排氣中CO2濃度，以降低CO2捕捉與封存成本，可解決二氧化碳引發之溫室效應問題，為目前效率最高之煤炭氣化技術之一，因此受到全世界之重視。

- 本公司正面臨能源價格高漲及二氧化碳排放所造成之營運壓力，高效率煤炭氣化技術係解決上述問題之重要對策之一，因此本次出國任務內容擬由原定之「節能低溫煙氣海水脫硫併脫氮整合新技術」變更為更有急迫性需求之「研習高效率煤炭氣化技術」，以加強綜研所在煤炭氣化之研發能力，並提供本公司電源開發處、環保處等單位電廠規劃設計之參考，及提供二氧化碳減量對策，以降低本公司因能源價格高漲及二氧化碳排放所造成之營運壓力。

- 美國俄亥俄州州立大學化學與生物分子工程系，擁有最先進之煤炭氣化化學環路程序技術及研發設備，因此至該大學研習高效率煤炭氣化技術

貮、出國行程

本次研研習高效率煤炭氣化技術之行程及工作概要如表2-1所示。

表2-1、實習行程概要表

| 日期 | 行程及工作概要 |
| --- | --- |
| 970518-970518 | 往程（台北－ 底特律 －哥倫布 ） |
| 970518-970529 | 美國俄亥俄州州立大學化學與生物分子工程系，研習高效率煤炭氣化技術。 |
| 970530-970531 | 返程（哥倫布－底特律－台北） |

參、工作記要

3.1研習煤炭直接氣化化學環路技術

俄亥俄州立大學致力於研究淨煤技術已經超過三十年，而在化學環路(Chemical Looping)上的研究也有超過五年的時間了，已經成功驗證小規模的直媒化學環路（CDCL）及合成氣化學環路（SCL）在發電、製氫和生產液體燃料過程中的應用。煤炭氣化化學環路程序是利用固體氧化物如氧化鐵進行氣化，可直接產生純的氫氣及二氧化碳，固體氧化物可以循環再利用，進而提升能源效率和減少排氣中二氧化碳之濃度，圖1為直煤化學環路之流程圖。

煤炭直接氣化化學環路系統之主要反應如下所示，可看出二氧化碳及氫氣是分開的，不需要進行困難之氣-氣分離，再加上可進行熱量回收，因此整体效率提高。氣体固体在燃料反應器內之反應如圖2所示。

圖2氣体固体在燃料反應器內之反應

在整体效率及發電效率上，煤炭直接氣化化學環路系統最高，其次為水煤氣化學環路（SCL）系統，各種煤炭氣化系統之效率如圖3所示。

傳統氣化系統需進行二氧化碳之分離，因此整体效率下降。

圖3 各種煤炭氣化系統之效率

3.2 研習合成氣化學環路系統（Syngas Chemical Looping, SCL）

- 合成氣化學環路系統係與傳統氣化爐結合如圖4 所示，傳統氣化爐產生之合成氣利用化學環路取代轉移(Shift reaction)，同樣可使二氧化碳及氫氣分開，其主要反應如下：

- Shift reaction

- CO + H2O = H2 + CO2

- Chemical looping

- CO + 2H2 + Fe2O3 = Fe + CO2 + 2H2O

- 3Fe + 4H2O= Fe3O4 + 4H2

化學環路系統可提升傳統氣化爐之效率，主要可節省Shift reactor及二氧化碳分離之耗能及成本。合成氣化學環路系統流程如圖4所示。

圖4合成氣化學環路系統流程

3.3 研習化學環路系統在煤炭製備液態燃料(coal to liquid, CTL)之應用

化學環路系統在煤炭製備液態燃料(coal to liquid, CTL)之應用，主要係利用化學環路系統將H2/CO比由0.5 提高至2，以提昇F-T 反應器製備液態燃料之效率。化學環路系統在煤炭製備液態燃料(coal to liquid, CTL)之應用流程如圖5所示。

圖5化學環路系統在煤炭製備液態燃料(coal to liquid, CTL)之應用流程

3.4 研習鈣化學環路系統之應用

鐵化學環路系統係利用鐵之氧化還原反應，製備氫氣及使得氫氣與二氧化碳分離，而鈣化學環路系統係利用氧化鈣吸收二氧化碳變成碳酸鈣之反應進行相關利用，主要利用有1.進行合成氣之除硫、製氫及二氧化碳分離純化反應如下圖6。2. 進行傳統粉煤鍋爐煙氣之除硫及二氧化碳分離純化反應。由於係氣固反應及可進行熱回收因此效率高。觀察重點在產生之硫化鈣如何再利用。

圖6 鈣化學環路系統

肆、感想與建議

4.1 感想

- 化學環路系統可應用發電、製氫、製備液壓燃料及二氧化碳之分離及純化，應用相當廣泛。

- 由於程序之簡化及低耗能操作使得化學環路系統效率遠高於傳統之煤炭氣化系統。

- 化學環路系統可適合各種燃料，包含各式各樣、不同等級之煤炭、燃油、燃氣及其他生質燃料等。

- 化學環路系統不需大量純氧、可節省大型空氣分離設備，裝置及操作成本降低。

- 化學環路系統具能源、環保及減碳優勢，係21世紀重要之淨煤技術。

- 化學環路系統是看似一簡單概念，唯程序要達到高效率，須在流体控制、觸媒製備及製程條件等精確要求下方能達成，需結合化工及材料等相關領域之專業知識。

4.2 建議事項：

建議主題:成立化學環路系統淨煤實驗室，為未來設置高效率及低碳淨煤電廠做好準備。

說明：

- 本公司燃煤電廠目前使用傳統粉煤鍋爐燃燒、發電，此系統發電效率不高且產生濃度約15-20% 含二氧化碳之煙氣，這種濃度之二氧化碳需進一步純化才能利用或封存。

- 本公司已規劃超臨界及未來超超臨界之發電機組，發電效率提高很多但仍產生濃度約15-20% 含二氧化碳之煙氣，需進一步純化才能利用或封存。

- 本公司須奉行行政院之政策綱領，發展高效率、低碳之淨煤發電技術。

作法：成立化學環路系統淨煤實驗室，發展化學環路相關技術，為未來設置高效率及低碳淨煤電廠做好準備。理由說明如下：

- 由於程序之簡化及低耗能操作使得化學環路系統效率遠高於傳統之煤炭氣化系統。

- 化學環路系統可應用發電、製氫、製備液壓燃料及二氧化碳之分離及純化，應用相當廣泛。

- 化學環路系統可適合各種燃料，包含各式各樣、不同等級之煤炭、燃油、燃氣及其他生質燃料等。

- 化學環路系統不需大量純氧、可節省大型空氣分離設備，裝置及操作成本降低。

- 化學環路系統具能源、環保及減碳優勢，係21世紀重要之淨煤技術。

- 成立化學環路系統淨煤實驗室，可瞭解相關原理及最佳操作條件，為未來設置高效率及低碳淨煤電廠做好準備，並發展新技術。

伍、附 錄

俄亥俄州立大學范良士教授相關淨煤技術專利及文獻

專利

PATENTS

- “Calcium Carbonate Sorbent and Methods of Making and Using Same,” with A. Ghosh-Dastidar and S. Mahuli, U.S. Patent 5779464 (1998).

- “Suspension Carbonation Process for Reaction of Partially Utilized Sorbent,” with S.Mahuli and R. Agnihotri, U.S. Patent 6309996 B1 (2001).

- “Carbonation Ash Reactivation Process and System for combined Sox and Nox Removal,” with R. Agnihotri, U.S. Patent 6569388 B1 (2003).

- “Method for the Treatment of Activated Carbonaceous Material Containing Alkali/Alkaline Earth Metals for the Reduction of NOx from Flue Gas,” with H. Gupta, U.S. Patent 6224839 B1 (2001); European Patent (Application No. 01 905 380.0-2104); Canadian Patent No. 2401279 (2006).

- “Neural Network Based Multi-Criteria Optimization Image Reconstruction Technique for Imaging Two- and Three-Phase Flow Systems Using Electrical Capacitance Tomography,” with W. Warsito, U.S. Patent  6,577,700 (2003).

- “Combustion Looping Using Composite Oxygen Carriers” with T. Thomas and P. Gupta U.S. Provisional Patent Series No. 11/010,648  (2004), U. S.  Patent Application Serial No. 11/010,648  (2004).

- “Carbon Dioxide Sequestration using alkaline Earth Metal-Bearing Minerals” with A. Park, U.S. Provisional Patent Series No. 11/027,368 (2004), Utility Patent, pending (2005).

- “Sulfidation and Regeneration of Metal Oxide Based Sorbents” with P. Gupta, U.S. Provisional Patent Series No. 60/611,209, September 17, (2004), Utility Patent, pending, (2005).

- “Process Flow Diagram for Calcium Based Reaction Separation for  CO2 (CaRS-CO2) Process”,  U. S. Provisional Patent 60/694,597  (2005).

- “Electric Capacitance Volume-Tomography” with W. Warsito, U.S. Provisional Patent Series No. 60/664,026 (2004), PCT WO 2006/102388 A1, International Publication Date: September 28, 2006.

- “High Temperature CO2 Capture Using Engineered Eggshells: a Route to Carbon Management”, with M. Iyer , U. S. Provisional Patent Series 60/662,690 (2005), U. S. Patent Application Publication  No. 2006/0211571 A1. (2006), International Application No. PCT/US2006/009880, (2006).

- “Regeneration of Calcium Sulfide to Mesoporous Calcium Carbonate Using Ionic Dispersants and Selective Reclamation of Unreacted Calcium from Calcium-Containing Solid Mixtures to Maximize Calcium Conversion and Prevent Recycling of Inerts” with H. Gupta and M. Iyer, U. S. Provisional Patent 60/694,702 (2005), Utility Patent, pending (No. 11/255,099), (2006); International PCT Application No. PCT/US2006/025493, (2006), International Publication No. WO 2007/002882 A2.

- Separation of Carbon Dioxide (CO2) from Gas Mixtures  by Calcium Based Reaction Separation (CaRS-CO2) Process” with H. Gupta, U.S. Patent No. 7,067,456 B2 (2006).

- “Separation of Carbon Dioxide (CO2) from Gas Mixtures  by Calcium Based Reaction Separation (CaRS-CO2) Proccess - Continuation-in-Part”  with H. Gupta and M. Iyer , U. S. Provisional Patent Series 11/255,099 (2005), U. S. Patent  Application No. 2006/0093540 A1 (2006), PCT Application No. PCT/US2006/025266, International Publication No. WO 2007/002792 (046815) A2.

- “"SiC Supported Sorbents for Use in Non-catalytic Gas-Solid and Gas-Solid-Solid Reactions", U.S. Provisional Patent Series No. 60/611,209, September 17, (2004), Utility Patent, pending,  11/229,054 (2005);(Can 2,520,081).

- “Electrostatic Tomography for Process Measurement and Control through Boundary Treatment”, with Q. Marashdeh,  US Provisional Patent , filed January 20, 2006.

- “Non-Invasive Electrical Resistive-Capacitive Tomography (ERCT) for Simultaneous Conductivity/Permittivity Imaging of Multi-Phase Flow”, with Q. Marashdeh, US Provisional Patent, filed January 20, 2006.

- “Methods for making Hydrogen from Gaseous and Solid carbonaceous Fuels”, with P. Gupta,  and L. Vargas, US Provisional Patent 60/758507, 2006, utility patent pending (2006).

- “Syngas Chemical Looping Process for synthesis of Chemicals and Fuels”,  with P. Gupta,  and L. Vargas, US Provisional Patent Series NO. 60/758,424 (2006), utility patent pending (2006).

- “Oxygen Carrying Particles for Hydrogen Production via Reduction-Oxidation Reactions and Its Synthesis Methods”, with P. Gupta and L. Vargas, US Provisional Patent, filed February 23, 2006, utility patent pending (2006).

- “Sub-Atmospheric (Vacuum) Calcination”, U. S. Provisional Patent 60/694,594 (2005).

- “Calcium Looping Process for High Purity Hydrogen Production integrated with in-situ Carbon Dioxide, Sulfur and Halide Capture in a Single Stage Reactor” with M. Iyer and S.  Ramkumar, U. S. Provisional Patent Application No. 60/826,809 (2006), utility patent pending (2007).

期刊論文

PUBLICATIONS

- “A Parametric Model for Evaporating Liquid Jets in Dilute Gas-Solid Flows” with C. Zhu, G.L. Liu and X. Wang.  Int. J. Multiphase Flow 28, 1479-1495 (2002).

- “Liquid Weeping and Bubbling Phenomena at Submerged Orifices: Part I: Experiments” with W.L. Peng and G.Q. Yang.  I&EC Research 41 (6), 1666-1677 (2002).

- “On the Second-Order Moment Turbulence Model for Simulating a Bubble Column” with L.X. Zhou, M. Yang, C.Y. Lian and D.J. Lee, Chem. Eng. Sci. 57, 3269-3281 (2002).

- “Gas and Solids Mixing in a Turbulent Fluidized Bed,” with B. Du, F. Wei, and W. Warsito, AIChE J. 48(9), 1896-1909 (2002).

- “Liquid Mixing in High Pressure Bubble Columns,” with G.Q. Yang, AIChE J. 49(8), 1995-2008 (2003).

- “Calcination and Carbonation Cycles Using Super Calcium Carbonate Sorbents for CO2 Separation from Flue Gases,” with H. Gupta, I & EC Research 41 (16), 4035-4042 (2002).

- Comments on “The Dynamics of Fluidized Particles” by R. Jackson, Cambridge Univ. Press, 2000, Int. J. Multiphase Flow 28, 525-526 (2002).

- “Bed Nonhomogeneity in Turbulent Gas-Solid Fluidization” with B. Du and W. Warsito, AIChE J. 49(5), 1109-1126 (2003).

- “Perspective: Clean Coal Technologies: OSCAR and CARBONOX Commercial Demonstrations” with R. Jadhav, AIChE Journal 48(10), 2115-2133 (2002).

- “Real time, 3-dimensional Imaging of Gas-Liquid-Solid Flows Using Electrical Capacitance Tomography with Neural Network Based Multi-criterion Optimization Image Reconstruction Technique” with W. Warsito, Physics of Fluids (in review, 2002).

- “Three-Dimensional, Discrete Simulation of Bubbling Phenomena in Gas-Liquid Bubble Columns and Gas-Liquid-Solid Fluidized Beds,” with C. Chen, AIChE J. 50(2), 288-301  (2004).

- “Reduction of Nitric Oxide from Combustion Flue Gas by Sub-bituminous Coal Char in the Presence of Oxygen” with H. Gupta, I & EC Research 42(12), 2536-2543 (2003).

- “ECT Studies on Choking Phenomena in a Circulating Fluidized Bed” with B. Du and W. Warsito, AIChE J. 50(7), 1386-1406 (2004).

- “A Semianalytical Expression for the Drag Force of an Interactive Particle Due to Wake Effect,” I&EC Research, 41, 5094-5097 (2002).

- “ECT Imaging of Three-Phase Fluidized Bed Based on Three-Phase Capacitance Model” with W.Warsito, Chem. Eng. Sci., 58, 823-832 (2003).

- “Neural Network Multi-Criteria Optimization Image Reconstruction Technique (NN-MOIRT) for Linear and Non-linear Process Tomography” with W. Warsito, Chem. Eng. and Processing J, 42 (8-9), 663-674 (2003).

- “ECT Velocimetric Technique for Three-Phase Fluidization Study” with W. Warsito, Canadian J of Chem. Eng. 81, 875-884 (2003).

- “CO2 Mineral Sequestration in a High Pressure, High Temperature Three-Phase Fluidized Bed Reactor” with A. Park, Canadian J of Chem. Eng. 81, 885-890 (2003).

- “Gas-Liquid Mass Transfer in High Pressure Bubble Columns” with R. Lau, W. Peng, L.G. Velazquez-Vargas, and G.Q. Yang, I&EC Research, 43(5), 1302-1311 (2004).

- “Reactive Separation of CO2 Using Pressure Pelletized Limestone” with H. Gupta, M. Iyer, B. Sakadjian, International Journal of Environmental Technology and Management,  4(1/2), 3-20 (2004).

- “Pilot Scale Studies of NOx Reduction by Activated High Sodium Lignite Chars: A Demonstration of CARBONOX Process” with H. Gupta, S. Benson, J. Laumb, E. Olson, C. Crocker, R. Sharma, R. Knutson, A. Rokanuzzaman and J. Tibbetts, I&EC Research , 43, No. 18, 5820-5827 ( 2004).

- “Turbulence Energy Spectra in Gas and Liquid Columns and Gas-Liquid-Solid Fluidized Beds” with Z. Cui, Chem. Eng. Sci., 59, 1755-1766 (2004).

- “On the Rise Velocity of an Interactive Bubble in Liquids” with J. Zhang, Chemical Engineering Journal, 93, 169-196 (2003).

- “Real-time ECT studies of choking phenomenon and flow behavior in a FCC riser” with W. Warsito and B. Du, Japanese J. of Multiphase Flow, Frontispiece, 17 (3), (2003).

- ”Electrical capacitance tomography imaging of gas-solid and gas-solid-liquid fluidized bed systems” with W. Warsito and B. Du, Journal of Visualization, Frontispiece, 7 (1), (2004).

- “Characteristics of Choking  Behavior in Circulating Fluidized Beds for Group B. Particles” with B. Du, I &  EC Research, 43, No. 18, 5507-5520 ( 2004).

- “Synthesis of High Surface Area SiC through modified Sol-Gel Rate: Control of Pore Structure” with P. Gupta and W. Wang, I & EC Research, 43, 4732 – 4739 ( 2004).

- “Multicyclic Study on the Simultaneous Carbonation and Sulfation of High Reactivity CaO: Effect of Residence Time on Selectivity towards Carbonation” with Mahesh V. Iyer, H. Gupta and Bartev S. Sakadjian, I & EC Research, 43, 3939 – 3947 (2004).

- “Advances in Gas-Liquid-Solid Fluidization” in Fluidization-XI, edited by U. Arena, R. Chirone, M. Miccio and P. Salatino, Engineering Conferences International Press, 1-20 (2004).

- “Simulation of an Oil Droplet on a FCC Particle in Feed Nozzle Conditions” with Y. Ge in Fluidization-XI, edited by U. Arena, R. Chirone, M. Miccio and P. Salatino, Engineering Conferences International Press, 259-266, (2004).

- “Three-Dimensional Simulation of  Impingement of a Liquid Droplet on a Flat Surface in the Leidenfrost Regime” with Y. Ge, Physics of  Fluids, 17, 027104, 1-20 ( 2005).

- “Gas-Liquid-Solid Fluidization: Perspectives” International Journal of Chemical Reactor   Engineering, Volume 2, p5, ( 2004).

- “CO2  Mineral Sequestration: Physically Activated Dissolution of Serpentine and pH Swing Process” with A. Park, Chemical Engineering Science, 59 , 5241 – 5247 ( 2005).

- “Bubble Modulation Using Acoustic Standing Waves in a Bubbling System” with Z. Cui, Y. Li, and Y Ge, Chemical Engineering Science, 60 (22), 5971-5981 (2005).

- “Dynamics of Spiral Bubble Plume Motion in the Entrance Region of Bubble columns and Three-Phase Fluidized Beds Using 3D ECT” with  W. Warsito, Chemical Engineering Science, 60 (22),6073-6084 (2005).

- “Effect of Nozzle Fan Angle on Sprays in Gas-Solid Riser Flow”, with Muhammad  M.R. Qureshi, C. Zhu, and C.-H. Lin,  Particuology, Vol. 4,  Nos. 3-4, 147-152 (2006).

- “3-D Modeling and Simulation of the Dynamics and Heat Transfer of a Subcooled Droplet Impacting on a Superheated Surface”, with Y. Ge, International Journal of Heat and Mass Transfer, Volume 49, Issues 21-23, pp. 4231 – 4249, (2006).

- “Imaging the Choking Transition in Gas-Solid Risers Using Electrical Capacitance Tomography” with B. Du and W. Warsito, , I &EC Research, 45, No. 15, 5384-5395, (2006).

- “A Second-Order moment Three-Phase Turbulence Model for  Simulating Gas-Liquid-Solid Flows”, Chemical Engineering Science,  60, 647-653 (2005).

- “Liquid Entrainment in High –Pressure Bubble Columns” with R. Lau and Z. Cui, I &EC Research, 44, No. 10, 3776 – 3782, (2005)

- “Electric Capacitance Volume Tomograpohy” with W. Warsito and Q. Marashdeh, IEEE Sensor Journal, Vol. 7, No. 4, 525 – 535 (2007).

- “Droplet-Particle Collision Mechanics with Film-Boiling Evaporation” with Ge Yang, Journal of Fluid Mechanics,  Vol. 573, 311-337 (2007).

- “ECT Studies of Gas-Solid Fluidized Beds of Different Diameters”  with B. Du and W. Warsito, I &EC Research, 44,No.14, 5020-5030, (2005).

- “Numerical Study of the Velocity Effects on the Impingement of an Acetone Droplet on a High –Temperature Surface” with Y. Ge, Journal of Chinese Institute of Engineering, Vol. 28, No.7, 1077-1087 (2005).

- “High Temperature CO2 Separation Using Waste Oyster Shells” with M. Iyer, A. Sparks, and T.Vonder-Haar, International Journal of Separation and Process Engineering, Japan 235 – 245, Vol.35,No. 4, (2005).

- “Nonlinear Forward Problem Solution for Electrical Capacitance Tomography Using Feed Forward Neural Network” with Q. Marashdeh, W. Warsito and F. Teixeira,  IEEE  Sensors Journal, Volume 6, NO. 2, 441-449, (2006).

- “Large-Eddy Simulation of Bubble-Liquid Confined Jets” with Yang Min and Zhou Lixing, Chinese Journal of Chemical Engineering, 10 (4) 381-384, (2002).

- “Droplet Size Effects on the Dynamics of the Impingement of an Acetone Droplet on a High-Temperature Surface” with Y. Ge, Journal of Chinese Institute of Chemical Engineering, Volume 37, No. 1, 71-80, (2006).

- “Electrostatic Charging Phenomenon in Gas-Liquid-Solid Flow Systems” with A Park, Chemical Engineering Science,  Vol. 62,  No. 1-2, 371-386  (2007).

- “Flow Dynamics of Gas-Solid Fluidized Beds with Evaporative Liquid Injection” with B. Du and W. Warsito,  Particuology – Science and Technology of Particles, Vol. 4, No. 1, 1-8 ( 2006) – cover article of the issue.

- "Effect of particle Size Ratio on the Drag Force of an Interactive Particle", with Zhang J., and J. P. Zhang,   Trans IChemE, Part A, Vol. 83(A4): 339-343 (2005).

- “Behavior of the Dense-Phase Transportation Regime in a Circulating Fluidized Bed” I&EC Research, 45, 3741 – 3751 (2006).

- “Characterization and Re-Use Potential of By-Products Generated from the Ohio State     Carbonation and Ash Reactivation (OSCAR) Process” with P. Taerakul, P. Sun, D. W.     Golightly, H. W. Walker, L. K. Weavers, B. Zand, T. Butalia, T. J. Thomas, and H.          Gupta,  Fuel 86 ,541-553 (2007).

- “Non-linear Image Reconstruction Technique for ECT Using a Combined Neural Network Approach” with Q. Marashdeh, W. Warsito, and  F. L. Teixeria, Measurement Science and Technology, 17 ,2097-2103 (2006).

- “A Multimodal Tomography System Based on ECT Sensors” with Q Marashdeh, W. Warsito, and F. L. Teixeira,  IEEE Sensor Journal, Vol. 7, No 3. 426 – 433 (2007).

- “Bubble Formation and Dynamics in Gas-Liquid-Solid Fluidization – A Review”, with G. Q. Yang and B. Du, Chemical Engineering Science, Vol. 62,  No. 1-2, 2-27  (2007).

- “Kinetics and Structural Characterization of Calcium-based Sorbents Calcined under Sub-atmospheric Conditions for High-Temperature CO2 Capture Process" with Bartev Sakadjian and M. Iyer,  I & EC Research 46 , 35-42 (2007).

- “A Moving Bed Reactor Setup to Study Complex Gas-Solid Reactions”, with P. Gupta,L. Vargas, C. Valentine,  Review of Scientific Instruments 78 085106 (2007).

- “3D Direct Numerical Simulation for Film-Boiling Contact of Moving Particle and Liquid Droplet” with Y. Ge, Physics of Fluids 18 , 117104 (2006).

- “Coal Cleans Up Its Act” with M. Iyer, The Chemical Engineer, PP.36-38, October, 2006.

- “SGR Process to Produce Hydrogen from Coal Derived Syngas” , with P. Gupta, and L. Velazquez-Vargas,  Energy and Fuels, Vol. 21, 2900 – 2908 (2007). “Multi-Scale Simulation of Oblique Collision of A Droplet on a Surface in the Leidenfrost Regime”, with Z. Yu and Y. Ge, Chemical Engineering Science, 62, 3462 – 3472 (2007).

- “Development of Electical Capacitance Volume Tomography (ECVT) and Electrostatic Tomography (EST) for 3D Density abnd Charge Imagiong of Fluidized Bed System”, with B. Du, Q. Marashdeh, W Warsitop and A Park,  in “Fluidization-XII” edited by Berruti, F., X. Bi., and T. Pugdley, Engineering Conference Institute,473 - 480 ( 2007).

- “Microparticle Flow in Liquid Medium: 3-D Confocal Microscopic Imaging and Computation of Flow in Microchannels”, with O Hemminger, Z Yu, and J. Lee, in “Fluidization-XII” edited by Berruti, F., X. Bi., and T. Pugdley, Engineering Conference Institute, 489 - 496 (2007).

- “Modeling on Heterogeneous Structure in Acceleration Regime of Gas-Solid Riser Flows” with J. You and C Zhao, in “Fluidization-XII” edited by Berruti, F., X. Bi., and T. Pugdley, Engineering Conference Institute, 119 - 128 ( 2007).

- “Calcium Reclamation and Synthesis of Precipitated Calcium Carbonate for Acid Gas Control in Flue Gas”, with I. Pryce, and A. Park , I&EC Research (in review, 2006).

- “Pilot Scale Demonstration of High-Temperature Multi-pollutant Control by Carbonated Fly Ash and Mesoporous Calcium Carbonate from Coal Combustion-based Flue Gas”,  with H. Gupta, T. J. Thomas, A.-H. A. Park, M. Iyer, P. Gupta, R. Agnihotri, and R. A. Jadhav, I&EC Research, 46, 5051 – 5060 (2007).

- “Case Study of Particle Technology Application in Automotive Industry: Novel Powder Feeding and Solution Drying Devices For Hydrogen Carrying Solid Fuel based fuel cell cars”, with A. Park, Chemical Engineering Education (in review, 2006).

- “Images Reconstruction of Objects in Ionized Gas Stream based on the Electrostatic Signals”, withy A. Park and Q. Marashdeh, IEEE Sensor Journal ( in review, 2006).

- “Bubbles in Nanofluids”, with O. Hemminger, Y. Zhao, and F. Wang,  I&EC Research, 46, 4341-4346 (2007).

- “Some Comments on “Spatial Imaging with 3D Capacitance Measurements by R. Wajman, R. Banaskiak, L. Mazurkiewicz, T. Dyakowski and D. Sankowski, Measurement Science and Technology Vol. 17, pp 2113-2118, 2006”, with W. Warsito and Q Marashdeh, Measurement Science and Technology, Vol. 18, 3665 – 3667 (2007). 2007).

- “Clean Coal”, with Fanxing Li,  Physics World,  Volume 20, No. 7 ( July  issue) 37-41 (2007).

- “Flow Regime Transitions in an Internal-Loop Airlift Reactor”, with Chun-Chong Fu and Wen-Teng Wu, Chem. Eng. Technol. 30 (8) 1-7 (2007).

- “Experiment and Lattice Boltzmann Simulation of Two-Phase Gas-Liquid Flows in Microchannels”  with  Zhao Yu; Orin  Hemminger, Chemical Engineering Science, 62, 7172-7183 (2007).

- “Electrical Capacitance Tomography - A Perspective” with Q. Marashdeh, B. Du, and W. Warsito, Industrial & Engineering Chemistry Research (in press).

- “Chemical Looping Gasification” with L. Vargas, F. Li and  S Ramkumar,  in Proceedings of 9th Circulating Fluidized Bed Conference, May 13-16, 2008; Proceedings edited by J. Werther .

- “Direct Simulation of the Buoyant Rise of Bubbles In Infinite Liquid using
 Level Set Method" with Z. Yu, Canadian Journal of Chemical Engineering, (in press, 2008).

- “Utilization of Chemical Looping Strategy in Coal Gasification Processes” with F. Li and S. Ramkumar,  Particuology,  ( in press, 2008).

- “Comparison of ECVT and MR Measurements of Voidage in a Gas- Fluidized Bed”, with  D. J. Holland, Q. Marashdeh, C. R. Muller, F. Wang, J. S. Dennis, and L. F. Gladden, I & EC Research, (in review).

- “Heterogeneous Structure in Gas-Solid Flows” with J. You, C. Zhu, and B Du, AIChE J. (in press).

“Droplet Collision with Porous Surface under Film Boiling Regime”, with Y. Zhao and F. Wang,  I&EC Research, (in press, 2008).