Proceedings of the Seventh International ASME Conference on Nanochannels, Microchannels and Minichannels
ICNMM2009
June 22-24, 2009, Pohang, South Korea

ICNMM2009-82037

The study of intelligent fuzzy weighted input estimation method combined with the experiment verification

Ming-Hui Lee1 Tsung-Chien Chen2 Tsu-Ping Yu3 1Department of Civil Engineering, Chinese Military Academy, Fengshan, Kaohsiung, Taiwan, R.O.C., g990406@mail.com 2Department of Power Vehicle and Systems Engineering, Chung Cheng Institute of Technology, National Defense University, Ta-His, Tao-Yuan, Taiwan, R.O.C., chojan@ccit.edu.tw

3School of Power Vehicle and Systems Engineering Chung Cheng Institute of Technology, National Defense University, Ta-His, Tao-Yuan, Taiwan, R.O.C., johinuee0921@hotmail.com

Abstract

The innovative intelligent fuzzy weighted input estimation method (FWIEM) can be applied to the inverse heat transfer conduction problem (IHCP) to estimate the unknown time-varying heat flux as presented in this paper. The feasibility of this method can be verified by adopting the temperature measurement experiment. The experiment modular may be designed by using 4 copper samples with different thicknesses. Furthermore, the bottoms of the samples are heated by applying the standard heat source, and the temperatures on the tops are measured by using the thermocouples. The temperature measurements are then regarded as the inputs into the presented method to estimate the heat flux in the bottoms of samples. The influence on the estimation caused by the processing noise covariance Q, the weighting factor , the sampling time interval , and the space discrete interval , will be investigated by utilizing the experiment verification. The results show that this method is efficient and robust to estimate the unknown time-varying heat input.

Keywords: Input Estimation Method, IHCP, Heat Flux.

introduction

Given the known initial conditions, boundary conditions, and physical properties of materials in the heat conduction problem, to investigate the temperature distribution in the solid by utilizing the heat-conducting equations is called the direct heat conduction problem (DHCP). On the other hand, the estimation of unknown heat flux, heat contact coefficient, heat conduction coefficient, and heat source by utilizing the temperature measurements inside the heat-conducting solid is called the inverse heat conduction problem (IHCP). The bottoms of samples are heated by applying the standard heat source, and the actual temperatures on the top are measured by using the thermocouples. The temperature measurements are then regarded as the inputs into the presented method to estimate the flux of the standard heat source. Furthermore, the feasibility of presented theory method can be verified through the procedure of this research.

The related researches about the inverse heat transfer experiment are as follows. Gardon and Akfirat [1] in 1965 presented the complete heat transfer measurement for the two-dimensional turbulence injecting shock. Linton and Agonafer [2] utilized the calculation of the hydromechanics software, Phoenics, to investigate the heat transfer of the plate-shaped heat sink. The simulation results were later compared with the actual experiment data. It showed that there was a large error between the simulation and actual experiment, but it was accepted in the industry applications since the similar tendency still existed. Zhan [3] in 1997 presented the finite difference method to estimate the high temperature of a cutter under the friction force due to the high speed cutting process. A metal bar was simulated as a cutter, and its temperature at any position was inversely estimated in the experiment. Wang [4] in 1998 combined the function specification method with the least-square scheme to inversely estimate the heat flux and temperature.

Nomenclature

|  | Sensitivity matrix |
| --- | --- |
|  | Gradient matrix |
|  | Sample specific heat () |
|  | Measurement matrix |
|  | Identity matrix |
| k | Time (discretized) |
|  | Sample thermal conductivity () |
|  | Kalman gain |
|  | Correction gain |
|  | Sensitivity matrix |
|  | Global conductance matrix |
|  | Total number of nodes |
|  | Total number of time steps |
|  | Filter’s error covariance matrix |
|  | Error covariance matrix |
|  | Process noise covariance |
|  | Heat flux |
|  | The unknown input estimated heat flux |
|  | Total average value of the heat flux |
|  | The average value of the ‘i’th sample’s heat flux |
|  | Estimated heat flux |
|  | Measurement noise covariance |
|  | Innovation covariance |
| t | Time |
|  | Sampling time |
|  | Temperature |
|  | Initial temperature |
|  | Measurement noise vector |
|  | State vector |
|  | Observation vector |
|  | Thermal diffusivity |
|  | Weighting factor |
|  | Input matrix |
| δ | Kronecker delta function |
|  | Density of the sample () |
|  | Heat-melting coefficient of the sample |
| σ | Standard deviation |
|  | State transition matrix |
|  | Coefficient matrix |
|  | Coefficient matrix |
| ω | Process noise vector |
|  | Sampling time interval |
|  | Discrete space interval |

Two different samples with six different curves of heat flux were adopted in the experiment. The inner temperature data of the samples were measured for the estimation of heat flux and temperature. Xu [5] in 2004 presented a hybrid method, which combined the Laplace transformation technique with the finite difference method, in coordination with the least-squares scheme. The inner temperatures of the standard samples were measured for the use of estimating the heat dissipation quantity of CPU heat sink equipped on these samples. Liu [6] in 2006 utilized the same numerical method and measured the inner temperature of the standard sample for the use of estimating the surface temperature and heat flux of the sample. The result was later used to estimate the heat dissipation quantity and average heat transfer coefficient of the CPU heat sink. Wang [7] in 2006 utilized the measurements to inversely estimate the unknown boundary condition and the physical property. The chip heated by different heat sources was simulated to present the rising temperature in the real operating condition.

Besides, the related researches about the modern estimation theory based on the Kalman filtering technique and the recursive least square algorithm are discussed as follows. Tuan [8] presented an adaptive weighted input estimation method, which combines the Kalman filter (KF) [9] with a recursive least square estimator (RLSE). The residual innovation sequence is generated by the Kalman filter and applied to the real-time recursive least square algorithm to estimate the unknown heat flux. The constant weighting factor is applied to the RLSE to emphasize the weights of the latest data. In order to improve the adaptation and estimation capability of the estimator, the adaptive weighting function is used to replace the constant weighting factor in 1998 [10]. Although the input estimates converge slowly in the initial time when the adaptive weighting function is used in the RLSE, the estimator has relatively better overall tracking performance when the unknown input is time-varying regardless of the influence of the measurement noise interference [11]. A non-destruction ballistic experimental method was established in 2006 to measure the temperature by using the thermocouple equipped on the outer wall of gun barrel during the firing process [12]. A feasibility investigation to estimate the heat flux is produced with regard to the estimation of the high temperature due to the rapidly burning propellant in the inner wall of the gun tube. The input is time-varying and may not be easily predicted. As a result, it is difficult to choose an adaptive and efficient weighting function. To resolve this situation, Chen et al. [13] in 2008 presented an intelligent fuzzy weighting function to replace the weighting factor,, of the RLSE.

Improving the weighting efficiency of the RLSE is essential, because the unknown input is time-varying and changes continuously. The adaptive weighting function takes any input variation into consideration. Therefore, the inverse method is developed to rapidly track the target and effectively reduce the effect due to the noise. In this paper, the feasibility of this method can be verified by adopting the temperature measurement experiment. 4 copper samples with different thicknesses are adopted in the experiment. The bottoms of samples are heated by applying the standard heat source. The thermocouples are used to measure the temperatures on the tops of samples. The temperature measurements are then regarded as the inputs into the presented method, which can estimate the heat flux in the bottoms of samples. The influence on the estimation caused by the processing noise covariance Q, the weighting factor , and the sampling time interval , will be investigated by utilizing the experiment verification.. The results show that this method is efficient and robust to estimate the unknown time-vary heat input.

experiment equipment

The entire experiment modular includes the signal source, the test samples, the sensors, the data acquisition device, and the computer module. The purpose of experiment is to inversely estimate the temperature and heat flux of the sample by using the temperature measurements of the sample surface. Therefore, the samples are heated by adopting the standard heat source, and the thermocouples (K type) are equipped on the sample surface. The structure chart of experiment devices are shown in Figure 1. The experiment devices and the samples used are illustrated as follows:

(1) The signal source:

The standard heat source generator with the maximum

output power of 200W is compatible with an alternating/direct current power source of 110 volt. The 30VDC power is series connected and can supply stable power to the heater. The standard heat source generator provides heat from its bottom layer. The inner wall and top of this device is insulated. It is a critical technique to perform the experiment in the insulated condition for the direct heat conduction problem. In addition, 5 holes with the diameter of 2mm have been punched on the top of the generator for the thermocouples to measure the surface temperatures of the test samples.

(2) The test samples:

4 copper samples with different thicknesses of 4mm,

5mm, 6mm, and 7mm are used. The following thermal properties are used in the calculation. , ρ=8940, and .

(3) Sensors:

The thermocouple is the NIST ITS-90 Type K. The temperature range is 0~, the error range is  and the error ratio is 0.01％. The error of the hardware and equipment fabricated can be considered as the model error.

(4) The temperature data acquisition device:

The device with the type of NI-9211 and the interface of

4 data transmission (NI USB-9162) is manufactured by the National Instruments Company and can be used to implement the signal acquisition, procedure and transformation. It is composed of the high performance measurement and control card, the signal process modular, the filter amplifier, and the electric charge amplifier.

(5) The computer module (including the software programs):

a. Intel processor 1.6G computer, the signal express software, and the Matlab programming language can be used to process the signal data.

b. The SIGNAL EXPRESS acquisition software: The software in coordination with the data acquisition system developed by the National Instruments Company can collect data from the subject system in real time. The sampling rate, the temperature range, the sampling time, the sensor type, the compensation of the cold junction, the frequency channel, and the record style to record the real-time signal of the system can be configured.

c. The presented method can be programmed by using the Matlab programming language. The temperature measurements are then regarded as the inputs into the method, which is to estimate the heat flux in the bottoms of samples.

Figure 1. The structure chart of the experiment devices.

MATHEMATICAL FORMULATION

The boundary condition at the position, , is assumed to be heat-insulated . By equipping the thermocouple sensor at the position, , and measuring the surface temperature of the copper sample, the heat-conducting model is formed as shown in Figure 2.

Figure 2. The heat-conducting model.

The heat-conducting governing equations are as follows:

（1）

（2）

（3）

（4）

（5）

where  represents that the temperature is a function of time, , and the position, .  is the initial temperature.  is the heat-conducting coefficient of the sample material.  is the heat-melting coefficient of the sample material. , the heat-diffusing coefficient.  is the measurement noise.  is the temperature measurement, which is assumed to be the Gaussian white noise with zero mean. By using the central differential method to disperse Equation (1) with respect to the space derivative, the following equation is obtained.

（6）

where ,the space interval, and . By setting , the boundary condition, Equation (3), at the position,,can be written as follows:

（7）

By substituting Equation (7) in Equation (6), the time derivative equation when  can be obtained as the following equation.

（8）

When , the equation can be presented as follows:

（9）

On the other hand, when , and , the boundary condition in Equation (4), can be presented as

follows:

（10）

By rearranging Equations (8), (9) and (10) along with a simulated noise input, the continuous-time state equation can be obtained as the following:

（11）

where is assumed to be the Gaussian white noise with zero mean, and it represents the modeling error. Furthermore,

（12）

（13）

（14）

The continuous-time state equation ( Equation (11)), can be discretized with the sampling time, . The discrete-time state equation and its relative equations are shown as follows.

（15）

（16）

（17）

（18）

（19）

In the equations above,  is the state vector.  is the state transition matrix.  is the input matrix.  is the definite input array.  is the processing error input vector, which is assumed to be the Gaussian white noise with zero mean and with the variance, .  is the Dirac delta function. The discrete-time measurement equation is shown below.

（20）

where  is the observation vector at the kth sampling time. The measurement matrix, .  is the measurement error vector, which is assumed to be the Gaussian white noise with zero mean and with the variance, . After the state equation is obtained, the inverse estimation process is carried out by using the on-line input estimation method, which is the combination of the Kalman filter mechanism and the adaptive fuzzy weighting function of the recursive least square estimation (RLSE) algorithm.

the Intelligent Fuzzy Weighted RLSE Input Estimation Approach

The conventional input estimation approach has two parts: one is the Kalman filter without the input term, and the other is the fuzzy weighted recursive least square estimator. The system input is the unknown time-varying heat flux. The Kalman filter is operating under the setting of the processing error variance, , and the measurement error variance, R. It is to use the difference between the measurements and the estimated values of the system temperature as the functional index. Furthermore, by using the fuzzy weighted recursive least square algorithm, the heat flux can be precisely estimated. The detailed formulation of this technique can be found in Ref [14].

1. The equations of the Kalman filter are shown as follows:

（21）

（22）

（23）

（24）

（25）

（26）

（27）

2. The recursive least square algorithm:

（28）

（29）

（30）

（31）

（32）

（33）

（34）

is the estimated input vector.  is the error covariance of the estimated input vector. B(k) and M(k) are the sensitivity matrices.  is the correction gain.  is the bias innovation produced by the measurement noise and the input disturbance.  is the covariance of the residual.  is the weighting constant or weighting factor.

3. The construction of the intelligent fuzzy weighting factor:

The fuzzy weighting factor is proposed based on the fuzzy logic inference system. It can be operated at each step based on the innovation from the Kalman filter. It performs as a tunable parameter which not only controls the bandwidth and magnitude of the RLSE gain, but also influences the lag in the time domain. To directly synthesize the Kalman filter with the estimator, this work presents an efficient robust forgetting zone, which is capable of providing a reasonable compromise between the tacking capability and the flexibility against noises. In the recursive least square algorithm, is the weighting factor in the range between 0 and 1. The weighting factor  is employed to compromise between the upgrade of tracking capability and the loss of estimation precision. The relation has already been derived as follows (Tuan et al. 1998 [10]):

（35）

The weighting factor, , as shown in Equation (34) is adjusted according to the measurement noise and input bias. In the industrial applications, the standard deviation  is set as a constant value. The magnitude of weighting factor is determined according to the modulus of bias innovation, . The unknown input prompt variation will cause the large modulus of bias innovation. In the meantime, the smaller weighting factor is obtained when the modulus of bias innovation is larger. Therefore, the estimator accelerates the tracking speed and produces larger vibration in the estimation process. On the contrary, the smaller variation of unknown input causes the smaller modulus of bias innovation. In the meantime, the larger weighting factor is obtained according to the small modulus of bias innovation. The estimator is unable to estimate the unknown input effectively. For this reason, the intelligent fuzzy weighting factor for the inverse estimation method which efficiently and robustly estimates the time-varying unknown input will be constructed in this research.

The intelligent fuzzy weighted input estimation method is derived following as [13]:

The range of fuzzy logic system input, , may be chosen in the interval, . The input variable is defined as:

（36）

where .  is the sampling interval.  is the measure total time. The proposed intelligent fuzzy weighting factor uses the input variable  to self-adjust the factor  of the recursive least squares estimator. Therefore, the fuzzy logic system consists of one input and one output variables. The range of input, , may be chosen in the interval, , and the range of output, , may also be in the interval, . The fuzzy sets for  and  are labeled in the linguistic terms of EP (extremely large positive), VP (very large positive), LP (large positive), MP (medium positive), SP (small positive), VS (very small positive), and ZE (zero). The specific membership is defined by using the Gaussian functions.

A fuzzy rule base is a collection of fuzzy IF-THEN rules:

IF  is zero (ZE), THEN  is an extremely large positive (EP);

IF  is a very small positive (VS), THEN  is a very large positive (VP);

IF  is a small positive (SP), THEN  is a large positive (LP);

IF  is a medium positive (MP), THEN  is a medium positive (MP);

IF  is a large positive (LP), THEN  is a small positive (SP);

IF  is a very large positive (VP) THEN  is a very small positive (VS);

IF  is an extremely large positive (EP) THEN  is zero (ZE),

where  and  are the input and output of the fuzzy logic system, respectively. Therefore, the nonsingleton fuzzier can be expressed as the following equation:

（37）

decreases from 1 as  moves away from .  is a parameter characterizing the shape of .

The Mamdani maximum-minimum inference engine was used in this paper. The max-min-operation rule of fuzzy implication is shown below:

（38）

where c is the fuzzy rule, and d is the dimension of input variables.

The defuzzier maps a fuzzy set  in  to a crisp point . The fuzzy logic system with the center of gravity is defined below:

（39）

is the number of outputs.  is the value of the  output.  represents the membership of  in the fuzzy set . Substituting  of Equation (39) in Equations (30) and (31) allows us to configure an adaptive fuzzy weighting function of the recursive least square estimator (RLSE).

DISCUSSION OF THE EXPERIMENTAL MEASURE-MENT AND ESTIMATIMATION RESULTS

To verify the performance of the proposed method, a standard heat source is modeled. The heat flux in the bottom is estimated inversely by measuring the temperature on the top. The test sample is heated by the standard heat source with the fixed power. The copper test sample is heated in the bottom. The inner wall and the top of the environment are insulated. 4 copper samples with different thicknesses and the thermocouple sensor located on the top surface, , and measuring the surface temperature of the copper sample are used. The total time period, sec. The sampling interval, sec. The measurement temperature curves of different test samples are shown in Figure 3.

The measurement error of the thermocouple is approximately ±0.01% (with the measurement noise covariance, =). The space step,  (=10). The process noise covariance matrix, . The temperatures are measured on the tops of different samples with different thicknesses. Figure 4 shows that the heat flux in the bottom is estimated inversely by substituting the temperature data into the presented method. The estimation results demonstrate that the penetration delay of temperature may exist in the estimation process. The time to reach the thermocouples will be shorter as the thinner test sample is used; on the other hand, the time will be longer as the thicker one is used. Since the standard heat source is not in an absolutely insulated condition in the measurement process, in order to reduce the influence of the penetration delay of temperature, 100 data are averaged to ensure the accuracy of the heat flux estimation. The data are selected from the smoothest curve (50sec) of Figures 4a~4d. The average value of the estimated heat flux, , where the average value of the measured data (70~120sec) from the 4mm test sample, =4011.921 , the average value of the measured data (90~140sec) from the 5mm test sample, =4015.924 , the average value of the measured data (160~210sec) from the 6mm test sample, =4041.769 , and the average value of the measured data (180~230sec) from the 7mm test sample, =4073.378 . The total average value was estimated by the average value of all test samples:

=4035.748 .         （40）

The total average value of the heat flux is regarded as the heat source in the bottoms of the test samples. In this paper, the percentage error (PE) is used to verify the precision of the estimation model. The definition of the PE is described in Equation (41).

（41）

where  is the total average value of the heat flux.  is the average value of the ith test sample’s heat flux. , which represent 4 test samples with different thicknesses (). The result shows that the heat flux of different test samples can be estimated precisely by using the temperature measurements. The PE of each test sample can be shown as follows.  (),  (),  (), and （）. The measured data from the test sample () have the smallest error. It can be used to analyze and test the presented estimation method.

The Kalman filter is operating under the setting of the processing error variance, , and the measurement error variance, . It regards the renewal values produced by using the difference between the temperature estimates and the temperature measurements as the functional index, and utilizes the real-time least square algorithm to precisely estimate the heat flux. The measurement error variance  of the thermocouple is assumed as a given value. The processing error variance,  is adjusted between to . The temperature measurements are then regarded as the inputs into the presented method, which estimates the heat flux (160~210sec) in the bottom of sample ().The relative root mean square error (RRMSE) of the estimated result can be calculated and chosen as the optimal estimation parameters. The definition of the RRMSE is described in Equation (42).

（42）

where  is the total number of time steps.  is the actual heat flux.  is the estimated heat flux. The RRMSE values of the estimation results can be plotted as shown in Figure 5. It indicates that when the process noise variance,  increases, it will accelerate the estimation speed and produce the better estimation result. On the other hand, when the process noise variance,  decreases, the error covariance matrix will decrease, and the Kalman gain  will therefore decrease. The main reason is that the correction need is decreasing, and only the smaller Kalman gain  is needed to offer that correction. On the other hand, as the process noise variance  increases, the error covariance matrix will increase, which causes the Kalman gain  to increase. The main reason is that the correction need is increasing, and therefore the larger Kalman gain  is needed to offer that correction. As a result, the Kalman filter will have faster correction performance.

Figure 5 shows that as the process noise variance () increases gradually, the RRMSE value will decrease gradually. However, when the value of process noise variance is chosen too large (), the Kalman filter will produce severe oscillations, which cause the RRMSE value to increase. Therefore, the value of process noise variance  will be optimal since the RRMSE is relatively small. The three difference parameters, , and  are adopted in the inverse estimation of the heat flux. The result verifies the feasibility of the presented method.

The estimation results of the heat flux using the constant weighting factors (= 0.125, 0.525, 0.925) and the intelligent fuzzy weighting function are plotted in Figure 7. The intelligent fuzzy weighting factor  plays the role of the controller, which is employed to compromise between the tracking capability and the loss of estimation precision. When the constant weighting factor (0.125) is adopted, the estimation convergence is fairly rapid. The only issue is that the oscillations are also produced. When the constant weighting factor (0.925) is adopted, the estimation convergence is fairly slow. However, the oscillation issue in this case is relatively small. As a result, the value of constant weighting factor should be appropriately chosen to obtain better estimation performance, and it will not be easy. The simulation results demonstrate that the fuzzy weighted input estimation inverse methodology can resolve this issue and produce better results in tracking the heat flux.

The sampling interval of the data acquisition device is 0.5sec. The interpolation method is used to increase the samples. The influences produced by using different sampling time on the estimation results when  and  are shown in Figure 8. The two sets of chosen sampling time,  = 0.05 and 0.005sec. According to the figure, the estimation model produces the best results when the sampling time  is 0.05sec.

Figure 3. The curves of temperature measurements（）

Figure 4a. The estimated heat flux ()

Figure 4b. The estimated heat flux ()

Figure 4c. The estimated heat flux ()

Figure 4d. The estimated heat flux ()

Figure 5. The RRMSE vs. The different values of Q, ()

Figure 6. The estimation results of the heat flux    （,）

Figure 7. The estimation results of the heat flux.（=0.125,

0.525, 0.925 and the fuzzy weighting function）

Figure 8. The estimation results of the heat flux.（=0.5, 0.05, 0.005）

CONCLUSIONS

In this paper, the bottoms of the copper samples with different thicknesses are heated by applying the standard heat source, and the temperatures on the top are measured by using the thermocouples. The FWIEM is utilizing the measured temperature data to estimate the heat flux in the bottoms of samples. The influence on the estimation caused by the processing noise covariance , the weighting factor , the sampling time interval , will be investigated by utilizing the experiment verification. The results reveal that if the process noise variance  increases and the smaller weighting factor is adopted, the faster estimation convergence and larger oscillations will be produced. The experiment verification shows that the FWIEM has the properties of better targat tracking capability and more effective noise reduction, and that it is an efficient, adaptive, and robust inverse estimation method for the estimation of the unknown heat flux.

Acknowledgments

This work was supported by the National Science Council of the Republic of China under Grant NSC97-2221-E-606-029.

References

1. Gardon, R., and Akfirat, J. C., “The role of turbulence in determining the heat transfer characteristics of impinging jet,” International Journal of Heat and Mass Transfer, Vol. 8, pp. 1261-1272, 1965.

2. Linton, R. L., and Agonafer, D.,“Coarse and Detailed CFD Modeling of a Finned Heat Sink,”IEEE Transactions on Components Packaging and Manufacturing Technology A, Vol. 18, No. 3, pp. 517-520, 1995.

3. Zhan, S. Z., “Application of the Inverse Heat Conduction Analysis for a On-line Temperature Control System: Theory and Experiment,” Department of Mechatronic Engineering, Huafan University, Master Paper, 1997.

4. Wang, J. H., “Analysis and Experiment of Inverse Heat

Conduction Problem, Department of Mechatronic

Engineering,” Huafan University, Master Paper, 1998.

5. Xu, M. F.,“Application of the Inverse Method to Estimate Heat Dissip-ation Effects on CPU Heatsink with Experimental Data,” Department of Mechanical Engineering, National cheng kung University, Master Paper, 2004.

6. Liu, Y. Z., “Application of the Inverse Method to Estimate Heat transfer Coefficient on CPU Heatsink using Experimental  Temperature Data,” Department of Mechanical Engineering, National cheng kung University, Master Paper, 2006.

7. Wang, S. Y., “The Analysis and Research on Thermal Chip with Non-uniform Heat Sources,” Institute of Mechatronic Engineering, Nor-thern Taiwan Institute of Science and Technology, Master Paper, 2006.

8. Tuan, P. C., Fong, L. W. and Huang, W. T., “Analysis of On-Line Inverse Heat Conduction Problems,” Journal of Chung Cheng Institute of Technology ,25(1),pp.59-73,1996.

9. Kalman R. E., “a New Approach to Linear Filtering and Prediction Problems,” ASME Journal of Basic Engineering, Series 2D,pp.35-45, 1960.

10.Tuan, P. C. and Hou, W. T., “The Adaptive Robust Weighting Input Estimation for 1-D Inverse Heat Conduction Problem,” Numerical Heat Transfer, Part B, Vol.34, pp. 439-456, 1998.

11.Tuan, P. C., and Hou, W. T., “Adaptive Robust Weighting Input Estimation Method for the 1-D Inverse Heat Conduction Problem,” Numerical Heat Transfer, Part B, Vol. 34, pp. 1-18 (1998).

12.Chen, T. C., Hsu, S. J., “A Study of the Gun Barrel Temperature Measurement and Heat Flux Estimation Using Non-destruction Ballistic Experimental Method,” JOURNAL OF EXPLOSIVES AND PROPELLANTS, R.O.C., Vol.22, No.2, p20-21, 2006.

13.Chen, T. C. and Lee, M. H., “Intelligent fuzzy weighted input estimation method applied to inverse heat conduction problems,” International Journal of Heat and Mass Transfer, Vol.51, pp. 4168–4183, 2008.

14. Tuan, P. C., Ji, C. C., Fong, L. W., Huang, W. T., “An input estimation approach to on-line two dimensional inverse heat conduction problems, Numerical Heat Transfer B29 (1996) 345-363.